# Superior MotionCam HD (PhOD) Jeweller

| # Communication | Jeweller communication technology Proprietary wireless communication technology to transmit commands, alarms, and events. Key features: - Two-way communication. - Advanced sabotage protection. - Block encryption featuring a dynamic key. - Instant notifications. - Remote control and configuration in Ajax apps. Frequency bands 868.0–868.6 MHz Depends on the sales region. Maximum effective radiated power (ERP) up to 20 mW Automatic power control to reduce power consumption and radio interference. Radio signal modulation GFSK Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub (or a range extender) in an open space. Encrypted communication All stored and transmitted data is protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. TurboWings communication technology Proprietary wireless communication technology to transmit photo verifications. Key features: - Two-way communication. - Advanced sabotage protection. - Block encryption featuring a dynamic key. - Data packages reload in case of transmission errors. - Fast photo delivery. - Photo delivery confirmation. Frequency bands 868.0–868.6 MHz Depends on the sales region. Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub (or a range extender) in an open space. Encrypted communication All stored and transmitted data is protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. |
| --- | --- |
| # Detection | Sensitive element 1 PIR sensor Motion detection distance up to 12 m up to 39 ft When installed at a height of 2.4 m. When installed at a height of 7.9 ft. Motion detection angle horizontal: 88.5° Motion detection speed from 0.3 to 2 m/s from 1 to 6.6 ft/s The detector lens should be positioned perpendicular to the likely intrusion path into the facility. Sensitivity 4 levels Adjusted by PRO or a user with admin rights in an Ajax app Pet immunity weight: up to 20 kg weight: up to 44 lb height: up to 50 cm height: up to 1.6 ft Learn more > False alarm prevention SmartDetect Software algorithm. Temperature compensation Effective motion detection at temperatures from −10 °C to +40 °C. from 14 °F to 104 °F. |
| # Photo verification | Photo by Alarm The built-in camera is activated if the detector is armed and motion is detected. Photo on Demand The user takes pictures at any time or when the detector is armed (depending on settings). The feature is disabled by default. Photo by Scenario The detector’s camera takes pictures automatically when other Ajax detectors are triggered. The feature is disabled by default. Photo by Schedule The detector automatically takes photos at the specified time. The feature is not configured by default. Adjustable photo resolution 960 × 720 px 640 × 480 px (by default) 320 × 240 px 160 × 120 px Adjusted by PRO or a user with admin rights in an Ajax app. Camera viewing angles horizontal: 90° vertical: 50° HDR image Improves detail accuracy in bright and dark scenes. IR illumination For night photography and low-light conditions. Series of pictures from 1 to 5 images with a resolution of 320 × 240 px or 160 × 120 px from 1 to 3 images with a resolution of 640 × 480 px or 960 × 720 px Adjusted by PRO or a user with admin rights in an Ajax app. Photo delivery time TBC up to XX s — 160 × 120 px resolution up to XX s — 320 × 240 px resolution up to XX s — 640 × 480 px resolution up to XX s — 960 × 720 px resolution Photo delivery time depends on the image resolution selected in the device settings. |
| # Recommendations | It is recommended to install the detector at a height of 2.4 m. It is recommended to install the detector at a height of 7.9 ft. The device is designed for indoor use only. |
| # Sabotage protection | Tamper alarm Notifications on attempts to detach the detector from the surface or remove it from the mounting panel. Holding screw to secure the detector on SmartBracket Protection against spoofing via device authentication Communication loss detection after 36 s Detection time depends on the Jeweller or Jeweller/Fibra settings. |
| # Power supply | Batteries 2 × CR123A Pre-installed. Type type C Calculated battery life up to 4 years With Photo on demand toggle disabled. up to 3 years With Photo on demand toggle enabled. Operating voltage range 4.2−6.2 V⎓ Nominal operating voltage 6 V⎓ Quiescent current consumption at nominal voltage 0.030 mA Maximum current consumption at nominal voltage 400 mA Full capacity 1,550 mAh Low battery voltage 4.705–5.00 V⎓ Low battery recovery voltage 5.055–5.355 V⎓ End-of-life battery voltage 3.80 V⎓ Device is switched off. End-of-life battery capacity 150 mAh Manufacturer Panasonic |
| # Enclosure | Dimensions 135 × 70 × 60 mm 5.31" × 2.75" × 2.60" Weight of the white device 170 g 6.0 oz Weight of the black device 157 g 5.54 oz Operating temperature range from −10 °C to +40 °C from 14 °F to 104 °F Operating humidity up to 75% Protection class IP50 |
| # Colors | Black White |
| # Complete set | Superior MotionCam HD (PhOD) Jeweller SmartBracket mounting panel with holding screw 2 × CR123A batteries (pre-installed) Installation kit Quick start guide |
| Warranty | 24 months |