# SpeakerPhone Jeweller

Wireless voice module for alarm verification

| ### Devices compatibility | Hub 2 (4G) Jeweller With firmware version OS Malevich 2.24 or later. Hub Hybrid (4G) With firmware version OS Malevich 2.25 or later. |
| --- | --- |
| ### Communication | VoRF communication technology Proprietary wireless audio streaming protocol using time division duplex. Key features: - Two-way communication. - Advanced sabotage protection. - Full duplex audio communication. Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub in an open space. Frequency band 863.0–865.0 MHz Jeweller communication technology Proprietary wireless communication technology to transmit commands, alarms, and events. Key features: - Two-way communication. - Advanced sabotage protection. - Block encryption featuring a dynamic key. - Instant notifications. - Remote control and configuration in Ajax apps. Frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the sales region. Maximum effective radiated power (ERP) up to 25 mW Automatic power control to reduce power consumption and radio interference. Radio signal modulation GFSK Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub in an open space. Encrypted communication All stored and transmitted data are protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. Wings communication technology Proprietary wireless communication technology to update firmware. Key features: - Two-way communication. - Advanced sabotage protection. - Block encryption featuring a dynamic key. - Reloading the data packages in case of transmission errors. Frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the sales region. Radio signal range up to 1,700 m up to 5,550 ft Between the device and a hub in an open space. Encrypted communication All stored and transmitted data are protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. VoRF communication technology Proprietary wireless audio streaming protocol using time division duplex. Key features: - Two-way communication. - Advanced sabotage protection. - Full duplex audio communication. Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub in an open space. Frequency band 863.0–865.0 MHz |
| ### Key features | AI-powered sound optimization It provides: - Clear voice transmission. - Suppression of extraneous noise. - Improved audibility even at long distances. - Option to set up the AI feature in Ajax apps. Voice audibility up to 10 m Auto adjustment of the microphone sensitivity. Output speaker power 3 W Speaker volume up to 103 dB 83 dB at a distance of 1 m. Frequency range from 200 Hz to 16 kHz Audio codec Opus Remote firmware update When a new firmware is available, an admin or PRO with the rights to configure the system can run the update in Ajax apps. The hub must be connected to Ajax Cloud via Ethernet. |
| ### Recommendations | It is recommended to install the device at a height of 1.6 m. It is recommended to install the device at a height of 5.2 ft. The device is designed for indoor use only. |
| ### Sabotage protection | Tamper alarm Notifications on attempts to detach the device from the surface. Microphone cut-off detection Notifications of attempts to damage an in-built microphone. Holding screw To secure the device on SmartBracket. Protection against spoofing device authentication Communication loss detection after 36 s Detection time depends on the Jeweller settings. |
| ### Power supply | Battery 4 × CR123 Pre-installed. Calculated battery life up to 3 years In a standby mode. up to 12 hours In a conversation mode. Optional external power supply 12 V⎓, 1 A The power supply cable length should not exceed 10 m when using the 2 × 0.35 copper-conductor cable. When the external power supply is connected, the batteries operate as a backup power source. The power supply cable length should not exceed 32 ft when using the 2 × 0.35 copper-conductor cable. |
| ### Enclosure | Dimensions 112 × 112 × 42 mm 4.41" × 4.41" × 1.65" Weight 270 g 9.52 oz Operating temperature range from 0 °C to +40 °C from 32 °F to 104 °F Operating humidity up to 75% Protection class IP50 |
| ### Colors | Black White |
| ### Complete set | SpeakerPhone Jeweller SmartBracket mounting panel Installation kit Quick start guide |
| Warranty | 24 months |