# IndoorCam

Indoor Wi-Fi security camera with PIR motion detector and built-in AI

| ### Video stream | Camera 4 MP CMOS sensor Resolution up to 2560 × 1440 px Camera viewing angles horizontal: 110° vertical: 60° Video protocol JetSparrow Fast and encrypted peer-to-peer video protocol developed by Ajax Systems. Storage options Ajax Cloud (coming soon) NVR Video codec H.264 Dynamic range HDR ​​Improves the accuracy of detail in bright and dark scenes. |
| --- | --- |
| ### Communication channel | Wi-Fi 802.11 b/g/n module 2.4 GHz, up to 2 Mbit/s for the channel 2K video. Jeweller and Wings communication technologies Proprietary wireless communication technologies to transmit commands, alarms, events, and photo verifications. They serve as a backup communication channel when the device is added to the hub. |
| ### Key features | Two-way communication Clear and loud voice communication between users and the device. Motion detection The device detects movement using a built-in PIR sensor and software image frame analysis. Object detection using AI - People. - Pets. - Vehicles. Smart IR illumination up to 8 m up to 26 ft Automatic adjustment of intensity in real time to avoid overexposure. This allows you to see objects that are far away or too close to the camera in low-light conditions. Visual verification of Ajax alarms The system allows you to set up a scenario when Ajax detectors activate selected cameras to start recording. The result is immediately sent to the mobile or desktop monitoring app. Microphone For two-way communication and sound recording at the site. Speaker volume up to XX dB XX dB at a distance of 1 m. XX dB at a distance of 3.3 ft. Audio codec G722 Audio processing Always active noise suppression and echo cancellation. LED indication An LED indicator shows the actual state of the device. |
| ### PIR detection | Sensitive elements 1 × PIR sensor Motion detection distance up to 4 m up to 13 ft Depends on the sensitivity level adjusted in the device settings. Motion detection angle horizontal: 110° vertical: 60° Sensitivity 3 levels Adjusted by a PRO or a user with admin rights in Ajax apps. |
| ### Installation recommendations | It is recommended to install the device at a height of XX m. It is recommended to install the device at a height of XX ft. The device is designed for indoor use only. It is not recommended to install the device in direct sunlight. This may cause overheating and damage to the device's electrical components. |
| ### Sabotage protection | Holding screw secures the device on the mounting bracket mTLS Encrypted connection for data protection. Authentication via Ajax Cloud Without local access rights. Secure Boot Protection against firmware spoofing. OTA signature verification Secure automatic firmware update. Privacy Access rights distribution for users in mobile apps and for monitoring operators in PRO Desktop. Communication loss detection Wi-Fi: can be set from 30 s Jeweller/Wings: after 36 s Detection time depends on Ajax Cloud connection and Jeweller or Jeweller/Fibra settings. |
| ### Power supply | Power supply unit 12 V⎓ ± 20%, 1 А Device power consumption TBC |
| ### Enclosure | Dimensions 128 × 74 × 76 mm 5.04" × 2.91" × 2.99" Weight 235 g 8.29 oz Operating temperature range from 0 °C to +40 °C from 32 °F to 104 °F Operating humidity up to 90% with no condensation Protection class IP20 |
| ### Color | White RAL 9003 Black RAL 9005 |
| ### Complete set | IndoorCam Power supply unit Installation kit Quick start guide |
| Warranty | 24 months |