# DoorBell

Video doorbell with built-in AI, PIR sensor, and control via apps

| ### Video stream | Camera 4 MP CMOS sensor Resolution up to 2560 × 1440 px Camera viewing angles horizontal: 155° vertical: 90° Video protocol JetSparrow Fast and encrypted peer-to-peer video protocol developed by Ajax Systems. Storage options Ajax Cloud (coming soon) NVR Video codec H.264 Dynamic range HDR ​​Improves the accuracy of detail in bright and dark scenes. |
| --- | --- |
| ### Communication channel | Wi-Fi 802.11 b/g/n module 2.4 GHz, up to 2 Mbit/s. Jeweller and Wings communication technologies Proprietary wireless communication technologies to transmit commands, alarms, events, and photo verifications. They serve as a backup communication channel when the device is added to the hub. |
| ### Key features | Two-way communication Clear and loud voice communication between users and visitors near the device. Motion detection The device detects movement using a built-in PIR sensor and software image frame analysis. Object detection using AI - People. - Pets. - Vehicles. Smart IR illumination up to 6 m up to 20 ft Automatic adjustment of intensity in real time to avoid overexposure. This allows you to see objects that are far away or too close to the camera in low-light conditions. Visual verification of Ajax alarms The system allows you to set up a scenario when Ajax detectors activate selected cameras to start recording. The result is immediately sent to the mobile or desktop monitoring app. Microphone For two-way communication and sound recording at the site. Audio codec G722 Audio processing Always active noise suppression and echo cancellation. |
| ### PIR detection | Sensitive elements 1 × PIR sensor Motion detection distance up to 4 m up to 13 ft Depends on the sensitivity level adjusted in the device settings. Motion detection angle horizontal: 120° vertical: 100° Sensitivity 3 levels Adjusted by a PRO or a user with admin rights in Ajax apps. |
| ### Installation recommendations | It is recommended to install the device at a height of 1.2–1.5 m. It is recommended to install the device at a height of 4–5 ft. The device is designed for outdoor and indoor use. It is not recommended to install the device in direct sunlight. This may cause overheating and damage to the device's electrical components. |
| ### Sabotage protection | Tamper alarm Notifications of attempts to remove the device from the mounting panel. Holding screw secures the device on SmartBracket mTLS Encrypted connection for data protection. Authentication via Ajax Cloud Without local access rights. Secure Boot Protection against firmware spoofing. OTA signature verification Secure automatic firmware update. Privacy Access rights distribution for users in mobile apps and for monitoring operators in PRO Desktop. Communication loss detection Wi-Fi: can be set from 30 s Jeweller/Wings: after 36 s Detection time depends on Ajax Cloud connection and Jeweller or Jeweller/Fibra settings. |
| ### Power supply | Device power consumption 10 W Main DC voltage range 12–24 V⎓ Main AC voltage range 16–24 V~, 50/60 Hz Battery built-in rechargeable battery: Li-ion 600 mАh Intended to heat the device in a cold climate and provide a backup power supply for short periods. |
| ### Enclosure | Dimensions 145 × 47 × 34 mm 5.71" × 1.85" × 1.34" Weight 163 g 5.75 oz Operating temperature range from −25 °C to +60 °C from −13 °F to 140 °F Operating humidity up to 90% Protection class IP54 |
| ### Color | White RAL 9003 Grey RAL 7004 Graphite RAL 7024 Black RAL 9005 Grey RAL 7004 |
| ### Complete set | DoorBell SmartBracket mounting panel Wedge bracket (2 pcs) BellKit (for connection to a wired chime) Installation kit Quick start guide |
| Warranty | 24 months |