## NVR

Network video recorder

| Compatibility | Compatible hub models Hub Hub Plus Hub 2 (2G) Hub 2 (4G) Hub 2 Plus Hub Hybrid (2G) Hub Hybrid (4G) |
| --- | --- |
| Videostream | NVR models 8-channel 16-channel With up to 4K resolution (25/30 FPS). IP cameras - ONVIF (Profile S/T) - RTSP Storage hard drive with a capacity of up to 16 TB (SATA connectivity, 3.5” size) HDD not included. Decoding formats H.265 H.264 Bandwidth up to 100 Mbps |
| Recommendations | The device operating voltage is 110–240 V~. NVR is designed for indoor installation only. |
| Communication channels | Ethernet up to 100 Mbps Bluetooth BLE frequency band: 2.400–2.4835 GHz To set up an Ethernet connection. Radio signal power 2.5 mW (100 mW limit) |
| Key features | Motion detection NVR has built-in software to detect the motion in the frame. Possible to switch between image analysis by NVR or camera algorithms. In-app video management system - Videowall that combines images from all connected cameras. - Event filters. - Timeline with smooth preview. Visual verification of Ajax alarms NVR allows setting up a scenario in which Ajax detectors trigger specific cameras to start recording. This results straight in mobile and desktop (monitoring) apps. Privacy Access rights distribution for users in mobile apps and for monitoring operators in PRO Desktop. |
| Power supply | Main 110–240 V~, 50/60 Hz, 0.6 A Up to 24 W of mains power consumption. Backup CR2032 battery, 3 V Only to power the system clock. |
| Anti-sabotage | Tamper alarm Notifications about attempts to detach the device from the surface or remove it from the mounting panel. |
| Enclosure | Dimensions 251 × 251 × 51 mm/9.88″ × 9.88″ × 2.01″ Weight 700 g/24.69 oz Without a hard drive. Operating temperature range from 0°C to +40°C/from 32°F to 104°F Operating humidity up to 75% Protection class IP20 |
| Colours | Black White |
| Complete set | NVR SmartBracket mounting panel Power cable Ethernet cable Installation kit Quick Start Guide |
| Warranty | 24 months |