# NVR H2D16PAC (16-ch)

16-channel network video recorder with 4K HDMI output, 16 PoE ports, and support for 2 hot-swappable HDDs

| ### Video | Video input 16 channels Recording resolution 12 MP, 8 MP, 6 MP, 5 MP, 4 MP, 3 MP, 1080p, UXGA, 720p, VGA, 4CIF, DCIF, 2CIF, CIF, QCIF HDMI output resolution 3840 × 2160, 1920 × 1080, 1280 × 1024, 1280 × 720 px Live view / playback resolution 12 MP, 8 MP, 6 MP, 5 MP, 4 MP, 3 MP, 1080p, 720p, D1, VGA, CIF |
| --- | --- |
| ### Decoding | Format H.265, H.264, JetSparrow Live view window division templates: 1, 4, 9, 16, 25, 5+1, 7+1, 12+1 views custom: up to 10×10 views Synchronous playback 4 channels |
| ### Intelligent video analytics | Motion detection Ajax camera: 16 channels NVR: 16 channels NVR features built-in software to detect movement in the frame. It is possible to switch between image analysis by NVR or camera algorithms in an Ajax app. AI-powered object detection Ajax camera: 16 channels |
| ### Interface | HDD 2 × SATA III: supports hard drives with up to 16 TB capacity (2.5" and 3.5" form factors) Supports full hot-swap functionality. HDDs are not included. Network 1 × RJ45 self-adaptive Ethernet (10/100/1000 Mbps) 16 × 802.3af/at PoE RJ45 fast Ethernet (10/100 Mbps) USB rear panel: 2 × USB 2.0 (Type-A) For connecting a keyboard, mouse, or USB Audio Device Class devices (Audio IN/Audio OUT). front panel: 1 × USB 2.0 (Type-A) To connect a USB flash drive. |
| ### PoE standard | PoE port 16 × IEEE 802.3af/at (Alternative A, power over data pairs) Maximum power per port up to 25 W Total power budget up to 160 W |
| ### Network | Bandwidth incoming: up to 160 Mbps outgoing: up to 160 Mbps Protocol HTTP, HTTPS, TCP/IP, IPv4, IPv6, UPnP, SNMP, RTSP, UDP, SMTP, NTP, DHCP, DNS, DDNS Interoperability ONVIF |
| ### Key features | Supported devices - Ajax cameras - IP cameras via ONVIF (Profile S/T) or RTSP Redundancy mode Simultaneous recording of the same video streams to two storage devices to ensure data redundancy and reliability. Split mode Sequentially distributes video streams across multiple storage devices to balance the load and optimize storage. Built-in video management system - Integrated video wall interface with live camera feeds. - Smart event filters for quick access to relevant video recordings. - Interactive timeline with preview navigation for efficient playback and incident review. Visual verification of Ajax system alarms NVR supports a scenario in which events triggered by Ajax detectors automatically highlight the corresponding camera feed on the video wall. This allows an operator to immediately verify the intrusion. Privacy Advanced privacy controls ensure a high level of personal data protection. All access rights — including those for mobile app users, installers, and monitoring company operators via Ajax PRO Desktop — are fully configurable by the device owner. This guarantees that only authorized parties can access video streams, event history, or device settings in accordance with the owner's preferences. Accelerometer alarm when NVR is moved |
| ### Recommendations | The device's operating voltage is 100–240 V~. The device is designed for indoor installation only. |
| ### Power supply | Main 100–240 V, 50/60 Hz, 5 A Power consumption up to 280 W Clock battery CR2032 battery, 3 V Only to power the system clock. |
| ### Enclosure | Dimensions 330 × 330 × 44 mm 12.99″ × 12.99″ × 1.73″ Weight 3.3 kg 7.3 lb Without hard drives. Operating temperature range from 0 °C to +50 °C from 32 °F to 122 °F Operating humidity up to 75% Protection class IP20 |
| ### Color | Black White |
| ### Complete set | NVR H2D16PAC (16-ch) 2 × rack-mount brackets Power cable Ethernet cable Installation kit Quick start guide |
| Warranty | 24 months |